LiDAR Point-Cloud Segmentation and Completion

Results Overview

Ngo Vi Viet Anh — Master's Thesis · Results through July 17, 2026

# Summary

The project has a complete car-detection pipeline on SemanticKITTI (held-out full-sequence F1 0.79, precision 0.90) and, on top of it, a point-cloud completion system with the first quantitative real-data evidence that completion adds value. Three questions were carried to a definitive answer: (1) the ~0.74 recall limit is a hard ceiling of density-based clustering — root-caused to HDBSCAN splitting, confirmed by five negative mitigation studies; (2) the sim-to-real gap for the classifier is total and symmetric, so synthetic pretraining was dropped from production (kept as an ablation); (3) the long-standing “PCN produces blobs on real data” failure was an inference-normalization bug, not a domain-gap dead end — after the fix, completion measurably improves bounding-box estimates (BEV IoU 0.707 → 0.747) and reconstructs 30% of never-observed car surface within 10 cm, 7× a symmetry-mirror baseline, under a newly designed and validated metric.

# 1. The Pipeline

Per frame: preprocessing (z-filter, denoise, 0.05 m voxel downsample) → RANSAC ground removal → HDBSCAN clustering → geometric filtering → binary car/not-car classifier (dual-branch PointNet) → centroid tracking with track-level filtering → PCN completion of accepted car tracks. Seq 08 is held out from all training and tuning and is the honest evaluation sequence.

# 2. Detection Results

The pipeline was built up in four measured steps:

Geometric filter tuning. A per-cluster false-positive analysis (vegetation 38%, buildings 33%, trunks 13%) drove three threshold changes: precision 0.237 → 0.654 at −19% recall. The residual FPs are vegetation clusters that geometrically resemble cars — separable only by shape, which motivated the classifier. The shape thresholds do not limit car recall (they account for ~4% of rejections that touch a GT car; the rest are fragments of split cars, §3).

Learned classifier. Training on synthetic ShapeNet alone fails completely on real data (0 true positives — the first sign of the domain gap in §4). Mining 550k real clusters from SemanticKITTI with GT purity filtering and training on those lifted pipeline precision 0.654 → 0.969 at a 2-point recall cost. The task was narrowed to binary car/not-car after the 4-class version produced spurious motorcycle detections from 28 bus training samples.

Evaluation integrity. Seq 00 — the original headline sequence — turned out to be inside the classifier's training set. All headline evaluation moved to held-out seq 08; seq-00 numbers are kept only as a deterministic reference, marked as leaked.

Track-level filtering. A minimum-track-length + majority-vote filter cuts transient FPs and — unexpectedly — recovers detections that flicker between “car” and “unknown” across frames. Full-sequence F1 0.762 → 0.801.

| Evaluation | Precision | Recall | F1 | Mean IoU |
| --- | --- | --- | --- | --- |
| Seq 00, 100 frames (leaked; reference) | 0.984 | 0.761 | 0.859 | 0.942 |
| Seq 08, full 4,071 frames (held-out) | 0.903 | 0.699 | 0.788 | 0.895 |

The system is precision-saturated (~1 FP/frame) and recall-limited. The classifier threshold is not a lever on recall: cars are lost at clustering before the classifier ever sees them (lowering the threshold to 0.30 yields no recall gain), so the F1-optimal operating point is also the recall-maximal one.

Figure 1 — Bird's-eye-view detections on held-out seq 08.

# 3. The Recall Ceiling

Recall plateaus near 0.74 in every configuration, and we now know exactly why: 31–37% of GT cars are split across multiple HDBSCAN clusters (66% split rate at close range — large cars have internal density gaps), and the sub-fragments then fail the volume/point-count filters. The recoverable ceiling from cleanly-clustered cars is 63–68% — matching actual recall, i.e. the pipeline already finds essentially every car that clusters cleanly. Five mitigation strategies were implemented and all were negative on held-out data:

| Strategy | Outcome |
| --- | --- |
| BEV connected-component clustering | F1 0.779 vs 0.844 — 2D projection merges overlapping objects |
| Larger min_cluster_size | Wins on seq 00, loses on seq 08 — overfits |
| Post-clustering fragment merge | Absorbs walls/poles; net negative |
| Distance-adaptive HDBSCAN | Ring-boundary artifacts; worse than global |
| Temporal point accumulation | F1 collapses to 0.073 |

Framing for the thesis: this is a structural property of density-based clustering — it makes local decisions with no notion of objectness, so split fragments can never be re-assembled. The pipeline is not recall-competitive with end-to-end detectors; its value is that it needs no 3D box labels (semantic labels only), and it quantifies exactly where learning must take over — which is where the classifier (§2) and completion (§5) enter.

# 4. Sim-to-Real: Dropping Synthetic Pretraining

Two studies answered the “too-perfect synthetic data” concern. A controlled ablation (retrain from random init) showed the ShapeNet prior contributes nothing once 420k real clusters are available (macro F1 0.9285 scratch vs 0.9225 pretrained). The cross-domain matrix shows why — zero transfer in either direction, and fine-tuning completely forgets the synthetic domain:

| Train ↓ / Test → | Synthetic val | Real val (seq 08) |
| --- | --- | --- |
| Synthetic only (Stage A) | 0.999 (0.999) | 0.447 (0.000) |
| Real only (from scratch) | 0.304 (0.000) | 0.929 (0.885) |
| Synthetic → real fine-tuned | 0.318 (0.000) | 0.923 (0.875) |

Cell format: macro F1 (car F1). Note the synthetic-only model scores 0.807 accuracy on real data while finding zero cars — why we report macro F1 and confusion matrices, not accuracy.

After a full-scale re-check (scratch is neutral-to-better: seq 00 F1 0.844 → 0.859, seq 08 identical), the production classifier now trains on real data only. Stage A is kept as the thesis sim-to-real ablation. Synthetic data was only dropped for classification, where real labels are abundant — for completion (§5), where no real ground truth exists, it remains the training data and works once the right partiality pattern is simulated.

# 5. Point-Cloud Completion

## 5.1 The “blob” failure, solved

PCN trained on ShapeNet produced shapeless blobs on every real cluster, and three domain-adaptation attempts (noise augmentation, virtual Velodyne ray-casting, sparse-input training) failed — each modelled one mismatch axis (noise, scan geometry, density) but not the real one: the occlusion/partiality pattern. Two fixes resolved it. First, a KITTI-like single-view partial generator (matching real pipeline output) trains a PCN that completes cleanly in-distribution (Chamfer 0.16 m). Second — the actual headline — the blobs were largely an inference bug: the inference path applied PCA alignment and centroid/scale normalization never seen in training, degrading every checkpoint ~3.5×. With the corrected path, real clusters complete into car-shaped clouds. A by-product finding: accumulated-LiDAR pseudo-ground-truth is invalid for evaluating completion (it is itself one-sided, so Chamfer rewards under-completion) — which forced the new metric in §5.4.

## 5.2 Two supporting results

Input gate. Bad completions trace back to bad inputs — fragments and merged clusters leaked by clustering (§3). An L-shape-fit gate rejecting them raises plausible-car completion precision 38% → 69%. Cost: completion is attempted on ~64% of detected cars; gated tracks keep their raw points, so detection metrics are unaffected. The gated subset is not just easy cases — completion's largest gains are on the sparsest inputs (§5.3).

Architecture check. A transformer completer (PoinTr) trained on identical data has a small synthetic edge (~5% Chamfer) but is equivalent on real data (16/26 vs 18/26 plausible). Decoder capacity is not the bottleneck — inference geometry and the partiality gap are. PCN stays in production.

## 5.3 Completion improves the bounding box

Reference: amodal GT boxes accumulated from 40 well-observed static cars (with viewpoint-coverage and motion guards; dimensions match published car statistics). For 2,075 true-positive detections of these cars, boxes were fitted to the raw partial and to its completion with the same fitter. Per-car medians, Wilcoxon tests, n = 39:

| Metric | Raw | Completed | p | Verdict |
| --- | --- | --- | --- | --- |
| BEV IoU | 0.707 | 0.747 | 0.0019 | better |
| ¦ΔW¦ (m) | 0.270 | 0.170 | 0.00015 | better |
| ¦ΔH¦ (m) | 0.255 | 0.131 | 1.6e-10 | better |
| Center error XZ (m) | 0.286 | 0.234 | 2.8e-05 | better |
| ¦ΔL¦ (m) | 0.447 | 0.456 | 0.65 | neutral |
| Yaw error (deg) | 3.5 | 3.0 | 0.74 | neutral |

Completion helps most where the sensor saw least (BEV IoU on < 100-point inputs: 0.461 → 0.599). The one weak spot is length: the network does not extend the unobserved far end (signed ΔL −0.49 raw vs −0.55 completed; conservative bias, never too long). That is the current work item — plan in §6.

Figure 2 — Amodal GT box (black) vs raw-partial box (blue) vs completed box (green).

## 5.4 A valid real-data completion metric

Since pseudo-GT Chamfer is invalid (§5.1), a new metric was designed: complete a static car from one frame, then score coverage of donor points — observations of the same car from other frames — that lie ≥ 0.15 m from every input point, i.e. surface the input frame never saw. Baselines: the raw partial and a symmetry-mirrored partial. An out-of-GT-box guard catches hallucination. Same statistical design as §5.3:

| Method | Coverage@0.1 m | Median novel-dist (m) | Out-of-box |
| --- | --- | --- | --- |
| Raw partial | 0.000 | 0.518 | 0.000 |
| Mirrored partial | 0.043 | 0.332 | 0.008 |
| PCN completed | 0.304 | 0.161 | 0.000 |

All pairwise p < 1e-6; the metric passed its four pre-registered validation checks. This is the first real-data evidence that the network reconstructs genuinely unseen structure — 30% of never-observed surface within 10 cm, 7× the mirror baseline, with essentially zero hallucination — and it makes the far-end weakness directly measurable (coverage there: 0.133).

Figure 3 — Donor-frame metric: input partial, never-observed donor points, and method outputs.

# 6. Next Steps

Fix the far-end undershoot (active). Ordered by evidence: (1) inference geometry, no retraining — anchor the box at the observed L-shape corner and place the center longitudinally with a car-length prior (the current estimate corrects laterally but not longitudinally); (2) if needed, retrain with a visibility-weighted asymmetric Chamfer loss that makes omitting occluded structure cost more than conservative truncation; (3) contingency: fine-tuning on real partials, now measurable with the donor metric. Architecture changes are ruled out by the PoinTr result. Success criteria: far-end coverage (0.133) and signed ΔL (−0.545 m).

Amodal GT validity. Cross-validation against official KITTI tracklets is impossible — the raw drive behind seq 08 was never tracklet-annotated (verified on the KITTI download server); that absence is why the pseudo-GT was built. Validity rests on the paired design (both arms share the reference), construction guards, and the dimension sanity check.

Thesis writing. Pipeline, recall-ceiling, sim-to-real, and completion chapters; pipeline diagram.

# Appendix — Where to Find the Details

Every result above is backed by a numbered finding in the repository (docs/findings.md) with hypothesis, commands, and full tables. Mapping:

| Report section | Findings |
| --- | --- |
| §2 Detection pipeline | #2, #4–#14, #18, #20 |
| §3 Recall ceiling | #21–#24 |
| §4 Sim-to-real | #25, #30, #31 |
| §5.1 Blob failure → fix | #15–#19, #26 |
| §5.2 Gate; PoinTr | #27, #28 |
| §5.3 Box improvement | #29 |
| §5.4 Donor metric | #32 |

A longer companion document (results_report_2026_07_17_rev2.docx) contains the extended justifications: operating-point analysis, filter validity across vehicle sizes, gate cost accounting, and the full far-end remediation plan.